How to set Load user scalars

This will allow GW user interface and associated logging tools to show  Engine Load  in kPa
Example:

Using a Delco 315 absolute pressure sensor
Sensor Range 20 to 315 kPa

Output Range (volts) = Us ((P/437.5) – 0.0143) [.003181 P - .003529]

Since Us is a nominal 5.0 Volts.

A signal of 0volts = 20 kPa

A signal of 5volts = 315 kPa

Sensor range 295 kPa

This range is rescaled by options MAP min and MAP max.
So if MAP min = 0.06V and MAP max = 4.90V

The range of load will be:

min = 20 +  (295kPa × (0.06 / 5.00)  = 23.4 kPa

max = 20 +  (295kPa × (4.90 / 5.00)  = 309.1 kPa

For a system with an underlying (default) range of load 0-80%
Set Load scalar = (309-23) / 80 = 3.56 

And Load offset  = 23.3
Set Load Error Scalar = 3.56 
Then configure units from GW option Configure | Configure Units | Scaling to kPa
For a system with an underlying (default) range of load 0-100%

Set Load scalar =  (309-23) / 100 = 2.86 

And Load offset = 23.3

Set Load Error Scalar = 2.86

Then configure units from GW option Configure | Configure Units | Scaling to kPa 
Note since Boost Error is a relative measure it has no offset.

UNKNOWN SENSOR

[image: image1.wmf]y = 0.7853x - 0.3042

y = 0.7853x - 0.3042

-0.5

0

0.5

1

1.5

2

2.5

3

3.5

4

4.5

0

1

2

3

4

5

6

VOLTS

BAR


Here 0 volts gives a pressure of -0.3042 Bar Absolute
And 5 volts gives a pressure of 3.7 Bar Absolute
This range is rescaled by MAP min and MAP max

So if MAP min = 30 (Then configure units from GW option Configure | Configure Units | Scaling to kPa 0.586V) and MAP max = 240 (4.688V)

The range of load will be:

min = -0.304 +  (4.0Bar × (30/256)  = 0.16475 Bar A

max = -0.304 +  (4.0Bar × (240/256)  = 3.446 Bar A

For a system with an underlying (default) range of load 0-130

Set Load scalar = 0.03077 and Load offset = 0.0016475 for Bar

For a system with an underlying (default) range of load 0-100%

Set Load scalar = 0.0400 and Load offset 0.0016475 for Bar

� EMBED Microsoft Excel Chart ���








_234038968.xls
Chart1

		1.686

		2.28

		2.91

		3.58

		4.21

		4.85



VOLTS

BAR

1

1.5

2

2.5

3

3.5



Sheet1

		

		1		1.686		1

		1.5		2.28		1.5

		2		2.91		2

		2.5		3.58		2.5

		3		4.21		3

		3.5		4.85		3.5





Sheet1

		





Sheet2

		



VOLTS

BAR



Sheet3

		





		






