SUBARU96 ECU connection list

The layout of the connectors is viewed from the wire side looking in at the ECU. The

Largest plug has been named "D" Moving From left to right we then have "C","B","A".

-----~----- 	-----~----- 	-----~----- 	-----~-----

|D13|~|D01|	|C08|~|C01| 	|B06|~|B01| 	|A11|~|A01|

|D26|~|D14| 	|C16|~|C09| 	|B12|~|B07| 	|A22|~|A12|

-----~----- 	-----~----- 	-----~----- 	-----~-----

PIN 	NO. 	DESCRIPTION

A01 	132

A02 	131 	power 12v from main relay

A03 	52 	boost pressure sensor +5V (****Pin C of Map Sensor****)

A04 	51 	boost MAP pressure signal (****Pin B of Sensor****)

A05 	50 	*(intake air temp signal)******

A06 	49 	oxygen sensor signal

A07 	48 	Coolant water temp signal

A08 	47 	air flow sensor supply

A09 	46 	MAF air flow sensor signal

A10 	45 	air flow sensor 0V not used

A11 	121 	Earth control unit

B01 	38 	throttle sensor 0V

B02 	37 	throttle sensor sig

B03 	36 	throttle sensor 5V

B04 	35 	crank angle signal

B05 	34 	crank angle sensor 0V

B06 	33 	crank angle shield GND

C01 	24 	cam sensor signal

C02 	23 	cam sensor 0V

C03 	22 	cam sensor shield GND

C04 	21 	knock sensor screen

C05 	20 	knock signal

C06 	19 	idle switch not used

C07 	18 	serial comms RX

C08 	17 	serial comms TX

D01 	112 	idle speed control

D02 	111 	idle speed control

D03 	8	wastegate solenoid

D04 	7	Rad fan

D05 	6	eng diode main relay

D06 	5	purge ****(water injection /water spray relay)*****

D07 	4	igniter pin 1

D08 	3	igniter pin 2

D09 	2	igniter pin 3

D10 	1	igniter pin 4

D11 	103 	injector 3

D12 	102 	injector 2

D13 	101 	injector 1

-----~----- 	-----~----- 	-----~----- 	-----~-----

|D13|~|D01|	|C08|~|C01| 	|B06|~|B01| 	|A11|~|A01|

|D26|~|D14| 	|C16|~|C09| 	|B12|~|B07| 	|A22|~|A12|

-----~----- 	-----~----- 	-----~----- 	-----~-----

PIN 	NO. 	DESCRIPTION

A12 	134 	not used

A13 	133 	power 12v from main relay

A14 	60 	insulation check

A15 	59 	permanent power supply

A16 	58 	Barometric altitude sensor not used

A17 	57 	oxygen sensor screen 0V

A18 	56 	not used

A19 	55 	0v screen air meter

A20 	54 	detection of transmission not used

A21 	53 	sensors 0V barometric air (**** pin A of map sensor ****)

A22 	122 	earth

B07 	44 	not used

B08 	43 	atmos 2 sig

B09 	42 	parking switch Anti-Lag (PBW) switch active when grounded

B10 	41 	neutral switch not used

B11 	40 	speedo signal

B12 	39 	ign key switch

C09 	32 	*9 AC switch not used

C10 	31 	starter switch not used

C11 	30 	accel not used

C12 	29 	diagnosis read memory not used

C13 	28 	diagnosis test mode not used

C14 	27 	not used

C15 	26 	trouble code output not used

C16 	25 	engine speed output

D14 	114 	earth

D15 	113 	earth ignition

D16 	16 	Power steering swt

D17 	15 	Rad fan

D18 	14 	ti monitor not used

D19 	13 	malfunction light oil press warning not used

D20 	12 	press exchange solenoid

D21 	11 	********Wastegate invert******

D22 	10 	rad fan AC Relay

D23 	9	fuel pump relay

D24 	106 	inj earth

D25 	105 	inj earth

D26 	104	injector 4